Beam Plot

AC246 - Right Angle Bracket

Mounting / Alignment

Mount the sensor such that the surface of the object to be detected is approximately centered within the sensor’s sensing field.  Mount the sensor firmly to avoid vibration.  The sensor face should be parallel to the liquid or material surface and free of air currents.

For best results in sensing small objects, for sensors with a Far Limit more than 38.1 mm (1.50 in.), mount the sensor about 38.1 mm (1.50 in.) away from the object.

AC247 - Conveyor Rail Clamp / Bracket

Wiring Connections, Connector Model

Indicator LED’s

Amber LED:	ON when object is sensed.

Green  LED:	ON when object is not sensed.

Accessories

Model	AC130	                Cable, Straight, 4-Conductor Shielded - 5 meters (16 ft.)

Model      XSZCD102Y	Cable, Straight, 4-Conductor unShielded - 5 meters (16ft.)

Model 	AC246	                Mounting Bracket, Right Angle

Model	AC247	                Conveyor Rail Clamp/Bracket Set

Mounting Bracket Dimensions

General Specifications

Power Supply:

Supply:	+12 to 24 VDC (±10%) @ 25 mA max. (excluding output load)

Protection:	ESD and reverse-polarity

Sinking Output:

Maximum on-state voltage drop:	0.75 V @ 100mA

Maximum load current:	100 mA

Maximum applied voltage:	30 VDC

Protection:	ESD and over-current

Sourcing Output:

Maximum on-state voltage drop:	1.10 V @ 100mA

Maximum load current:	100 mA

Maximum output voltage:	Supply voltage - 1.10 volt @ 100mA

Protection:	ESD and over-current

Operating Temperature: 	-30C to 70C (-22F to 158F) @ 12v supply

-30C to 65C (-22F to 149F) @ 24v supply

Response Time: 	4 mS on / 4 mS off

Sensing: [TA=20C (68F)] Large Flat Target

Range:	76.2 mm (3.00 in.)

Field:	6.4 to 76.2 mm (0.25 to 3.00 in.)

Maximum plane-reflector angle:	 8

Sonic Cone Angle:	See beam plot

Window-edge accuracy:	 3.14 mm (0.124 in.)

Minimum object size   Rod: 	2.5 mm (0.098 in) at 38.1 mm (1.50”) range, 0 tilt

Large Flat Object:	1.0 mm (0.039 in) at 38.1 mm (1.50”) range, 0 tilt

Sensor Dimensions: 	33 x 19.1 x 7.6 mm (1.30 x 0.75 x 0.3 in.)

Mounting Hole Spacing:	13 mm (0.512 in.)

Mounting Hole Diameter	3.51 mm (0.138 in.)

Sensor Cable:	12 mm Micro Pigtail - 152mm (6.0 in.) length

(Must be purchased separately)	AC130 Straight, 4-conductor, 5 meters (16 ft.)
Sensor Materials:

Housing:	PEI

Transducer face:	Epoxy

Cable:	Non-toxic PVC jacket

LED Lens:	Polycarbonate

Sensor Ratings and Approvals:	NEMA 4X (Indoor Use Only) 5, 12, 12K, 13, and IP67

Installation/Overvoltage Category:	II

CE Mark Compliant:	Declaration of conformity available upon request.

This Product is UL Listed if powered by a Class II Power Supply and protected by an 0.8A Max UL Listed Fuse

LIMITATIONS AND EXCLUSION OF WARRANTIES

All goods purchased from Hyde Park Electronics LLC shall be free from defects in materials, design and workmanship under normal conditions of use for one year from the date of shipment.  THIS WARRANTY IS THE SOLE WARRANTY AND IS EXPRESSLY IN LIEU OF ALL OTHER WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY IMPLIED WARRANTY OF MERCHANTABILITY OF FITNESS FOR A PARTICULAR PURPOSE.  THE LIABILITY OF HYDE PARK TO ANY PURCHASER SHALL BE LIMITED EXCLUSIVELY TO THE COST OF REPLACEMENT OR REPAIR OF DEFECTIVE PARTS, AND SHALL NOT INCLUDE LIABILITY FOR ANY DIRECT, CONSEQUENTIAL OR INCIDENTAL DAMAGES WHATSOEVER, WHETHER FORESEEN OR UNFORESEEN, INCLUDING BUT NOT LIMITED TO LOST PROFITS, LOST SALES, OR INJURY TO PERSONS OR PROPERTY.

HYDE PARK ELECTRONICS LLC

1875 Founders Drive

Dayton, Ohio  45420-4017

Phone (937) 252-2121   Fax (937) 258-5830

Email: help@sesensors.com

Web Site: http://www.sesensors.com

 2001-2008 Hyde Park Electronics LLC

Proximity Sensor, Flat Profile, Ultrasonic

Normally Open Outputs

Sensing Field 76.2 mm (3.00 in.) from Sensor Face

OPERATOR INSTRUCTIONS

SUPERPROX

SM380A-344-00FP

This self-contained, miniature ultrasonic proximity sensor is capable of sensing most objects within a 76.2 mm (3.00 in.) sensing field (Fig.1).  Objects that are transparent, opaque, plastic, glass, metal, liquid or solid can be detected if located within the sensing field.

Range

76.2 mm

(3.00 in.)

6.4 mm

(0.25 in.)

0.0 mm

(0.0 in.)

Figure 1

Literature and application engineering assistance are provided by Hyde Park and its authorized distributors to aid the customer in selecting the product for an application.  The customer, however, is responsible for determining the suitability of the product in the application.