Beam Plot

Mounting / Alignment

Mount the sensor so that the sensor face is parallel to the liquid or material surface and free of air currents.  Mount the sensor firmly to avoid vibration.

Wiring Connections, Cable Model

Indicator LED

Amber LED:	Intensity indicates the magnitude of the analog output.

Accessories

Model 	AC226		Conveyor-Rail Clamp/Bracket Set

Model 	AC227		Mounting Bracket, Large (ACxxx Retrofit)

Model 	AC228		Mounting Bracket, Small

Mounting Bracket Dimensions

General Specifications

Power Supply:

Supply:	+ 15 to 24 VDC @ 50 mA max

Protection:	ESD and reverse-polarity

Analog Output:

Output Voltage:	0 to 10 Volts

Output Resolution:	9.77 mV (millivolts)

Output Load:	1000 ohms (minimum)

Protection:	Short-circuit to common and ESD

Response Time:	2.5 mS for full scale change

Operating Temperature:

@ 15 VDC supply	0 to 60C (32F to 140F) @ 100% relative humidity

@ 24 VDC supply	0 to 50C (32F to 122F) @ 100% relative humidity

Sensing:

Fixed window width:	69.9 mm (2.75 in.)

Fixed inner-window edge:	31.8 mm (1.25 in.)

Fixed outer-window edge:	101.6 mm (4.00 in.)

Resolution, object position, worst case:	 0.69 mm (0.027 in.)

Repeatability:	 0.69 mm (0.027 in.)

Minimum object area, flat smooth:	40 mm2 (0.027 in.) @ 152 mm (6.0 in.) distance

Minimum object diameter:	1.6 mm (0.06 in.) @ 152 mm (6.0 in.) distance

Maximum plane-reflector angle:	 10

Sonic Cone Angle:	7

Sensor Dimensions:

Threads:	18 mm x 1 mm

Length:	64.7 mm (2.55 in.)

Sensor Cable Length:	3 Meters (10 Ft.) Standard

Sensor Materials:

Housing:	Stainless Steel

Transducer face:	FDA approved silicone rubber

Cable:	Non-toxic PVC jacket

LED:	Polycarbonate

Sensor Environmental Ratings:

NEMA 5, 12, 12K, 13, and IP67

Installation/Overvoltage Category:	II

CE Mark Compliant:	Declaration of conformity available upon request.

LIMITATIONS AND EXCLUSION OF WARRANTIES

All goods purchased from Hyde Park Electronics LLC shall be free from defects in materials, design and workmanship under normal conditions of use for one year from the date of shipment.  THIS WARRANTY IS THE SOLE WARRANTY AND IS EXPRESSLY IN LIEU OF ALL OTHER WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY IMPLIED WARRANTY OF MERCHANTABILITY OF FITNESS FOR A PARTICULAR PURPOSE.  THE LIABILITY OF HYDE PARK TO ANY PURCHASER SHALL BE LIMITED EXCLUSIVELY TO THE COST OF REPLACEMENT OR REPAIR OF DEFECTIVE PARTS, AND SHALL NOT INCLUDE LIABILITY FOR ANY DIRECT, CONSEQUENTIAL OR INCIDENTAL DAMAGES WHATSOEVER, WHETHER FORESEEN OR UNFORESEEN, INCLUDING BUT NOT LIMITED TO LOST PROFITS, LOST SALES, OR INJURY TO PERSONS OR PROPERTY.

HYDE PARK ELECTRONICS LLC

1875 Founders Drive

Dayton, Ohio  45420-4017

Phone (937) 252-2121   Fax (937) 258-5830

Email: help@sesensors.com

Web Site: http://www.sesensors.com

 1997-2008 Hyde Park Electronics LLC

Ultrasonic Analog Output Sensor

18 mm Diameter

Inverse 0-10 Volt Analog Output

Fixed Sensing Window, 69.9 mm (2.75 in.) Wide

Window Located 31.8 to 101.6 mm (1.25 to 4.00 in.) from Sensor Face

OPERATOR INSTRUCTIONS

SUPERPROX

SM606A-444-00S

This self-contained, ultrasonic analog output sensor is capable of sensing most objects within a fixed 19.1 to 172.7 mm (0.75 to 6.80 in.) sensing range (Fig.1).  Objects that are transparent, opaque, plastic, glass, metal, liquid or solid can be detected if located within the sensing window.

Near Limit

31.8 mm

(1.25 in.)

Far Limit

101.6 mm

(4.00 in.)

19.1 mm

(0.75 in.)

0.0 mm

(0.0 in.)

Range

172.7 mm

(6.80 in.)

Figure 1

Literature and application engineering assistance are provided by Hyde Park and its authorized distributors to aid the customer in selecting the product for an application.  The customer, however, is responsible for determining the suitability of the product in the application.