Mounting / Alignment

|

Mount the sensor so that both transmitter and receiver are aligned opposite each other.

The Receiver’s amber LED is ON when sonic energy is detected from the transmitter.

Wiring Connections, Cable Model

Accessories

2

|

Model 	AC226		Conveyor-Rail Clamp/Bracket Set

Model 	AC227		Mounting Bracket, Large (AC201 Retrofit)

Model 	AC228		Mounting Bracket, Small

Mounting Bracket Dimensions

General Specifications

3

Power Supply:

Supply Voltage:	+12 to 24 VDC

Receiver Current:	40 mA max (not including load)

Protection:	ESD and reverse-polarity

Sinking Output:

Maximum on-state voltage drop @ 100mA:	0.75 V

Maximum load current:	100 mA

Maximum applied voltage:	30 VDC

Protection:	ESD and over-current

Sourcing Output:

Maximum on-state voltage drop @ 100mA:	1.10 V

Maximum load current:	100 mA

Maximum output voltage:	Equal to supply voltage

Protection:	ESD and over-current

Response Time: 	0.4 mS on / 0.4 mS off

Temperature:

Operating Temperature:	0C to 60C (32F to 140F), @ 100% relative humidity

Storage Temperature:	-10C to 70C (14F to 158F)

Sensing Range: 	See table in operation figure

Dimensions:

Threads:	18 mm x 1 mm

Length:	64.7 mm (2.55 in.)

Cable: 	3 Meters (10 Ft.) Standard

Materials:

Housing:	Stainless Steel

Transducer face:	Silicone-rubber

Cable:	Non-toxic PVC jacket

LEDs:	Polycarbonate

Environmental Ratings:

NEMA 4X (Indoor Use Only) 5, 12, 12K, 13, and IP67

Installation/Overvoltage Category:	II

CE Mark Compliant:	Declaration of conformity available upon request.

This Product is UL Listed if powered by a Class II Power Supply and protected by a 2.0A Max UL Listed Fuse

LIMITATIONS AND EXCLUSION OF WARRANTIES

All goods purchased from Hyde Park Electronics LLC shall be free from defects in materials, design and workmanship under normal conditions of use for one year from the date of shipment.  THIS WARRANTY IS THE SOLE WARRANTY AND IS EXPRESSLY IN LIEU OF ALL OTHER WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY IMPLIED WARRANTY OF MERCHANTABILITY OF FITNESS FOR A PARTICULAR PURPOSE.  THE LIABILITY OF HYDE PARK TO ANY PURCHASER SHALL BE LIMITED EXCLUSIVELY TO THE COST OF REPLACEMENT OR REPAIR OF DEFECTIVE PARTS, AND SHALL NOT INCLUDE LIABILITY FOR ANY DIRECT, CONSEQUENTIAL OR INCIDENTAL DAMAGES WHATSOEVER, WHETHER FORESEEN OR UNFORESEEN, INCLUDING BUT NOT LIMITED TO LOST PROFITS, LOST SALES, OR INJURY TO PERSONS OR PROPERTY.

HYDE PARK ELECTRONICS LLC

1875 Founders Drive

Dayton, Ohio  45420-4017

Phone (937) 252-2121   Fax (937) 258-5830

Email: help@sesensors.com

Web Site: http://www.sesensors.com

 1997-2008 Hyde Park Electronics LLC

.

18 mm Diameter Thru-Beam Receiver

Cable Style, Normally Open Outputs, Fast Response

Compatible Transmitters: SM8X0A-24-XXS, SM8X0A-40-XXS

OPERATOR INSTRUCTIONS

MICROSONIC

SM801A-40-01S

This Hyde Park ultrasonic, thru-beam receiver, when paired with a compatible transmitter is capable of sensing nearly any object, even transparent materials such as glass, plastic, and clear film.  Objects as small as 12.5 mm (0.50 in.) diameter or larger can be detected at speeds in excess of 2000 parts per minute.  Both a sinking (NPN) output and a sourcing (PNP) output are standard.

Figure 1.

Indicator LED’s

Amber LED:	ON when sonic beam is detected.

Literature and application engineering assistance are provided by Hyde Park and its authorized distributors to aid the customer in selecting the product for an application.  The customer, however, is responsible for determining the suitability of the product in the application.